Training program for PIRUM, version 930101

Copy TOPAZ.SSQ to T.SSQ

Below > is used to show commands given by you

>PIRUM

>INFO

Read carefully and use the RETURN commands until you will see

the list containing the commands.

After the main level prompter PIRUM> you may alway use the command

>HE

to display the command list

>QREAD

Spacing-data from file    Yes

>Y

You will be asked for the file name (i.e. T.SSQ)

The first line is a title line (Default is Yes) >RETURN

>CONV

and you will see the allowed kinds of input data.

Give 1 in this case as input data are sine square theta values

>SYS

Select the orthorhombic system (i.e. 3)

>CELL

You may answer that you will give direct cell parameters

and give 4.65 8.80 8.39

>COND

>HE

and you will see allowed command list

>INFO

and you will see allowed extinction conditions (except for centering)

>INP

>HE

note how centering should be given

Give the extinction conditions...

>h0l  :  h+l=2n

and

>0kl  :  k=2n

input is ended by an extra return

>EXIT

and you are back at the main level (as shown by the prompter PIRUM)

>CREATE

and give T.PIR as a file name.

This should be used in order to simplify a second run from start

You may use PREAD to read this file if IP=0 or IP=1 in a new run.

>DI

may be used to see the first test only on the screen

>RUN

(If your data set is less accurate than this one, it may be necessary

to increase DT)

In this example everything will probably look o.k. so you may set

>IP

to 1

This will in the run create an indexed reflection file

>DI

will toggle to a list file

>RUN

You will be asked to create a list file and a reflection file

(i.e.) the names T.LST and T.REF may be given

>WT

as a comparison you may set it to 1 (Explained in the PIRUM.DOC file)

>IP

to 0

as you may not need a new T.REF file

>RUN

give a new list file name (i.e.) TT.LST

>EXIT

Now you may compare T.LST and TT.LST

Usually the differences between the standard deviations may be larger.

It is of course dependent on the data.

Look at the T.REF file and as a first test you may accept all (hkl)s

>PIRUM

>IP

to 2

(do the requested return)

>PREAD

give the file name T.REF

>SYS

give 3 (as the cell is orthorhombic)

(In this test we assume that no extinction conditions are known)

>DI

on display

>RUN

o.k.

>DI

on file

>RUN

You may chose to overwrite the old list file T.LST (or create a new one)

Give T1.LST (and in a second run where the extinction conditions are

given T2.LST.)

>EXIT

Look at the list file T1.LST

You may see that  (413) is used as indices for a reflection where (207)

is (marginally) more close to the observed data. If you have any

information from structural data about intensities you will now have a

possibility to choose. Otherwise it may be wise to exclude the reflection

from the T.REF file in a second run (...and report the reflection as

double indexed in the final report).

In this example, however, there is no problem as only (h0l) with h+l=2n

is allowed. Thus, you may put in the extinction conditions as in the first

run described above and run this again. Use the commands COND followed

by INP after the SYS command. Give T2.LST as your new list file name.

Note that on T1.LST and T2.LST cell parameters and standard deviations

are identical. The figures of merit, however, are different as the number

of allowed calculated reflections are not the same. (The correct figures

are found on T2.LST.)

Now we assume that the sine square data for the strongest line (with the

intensity reported as 100) has been measured erroneously.....

Use your editor to change the sine square value 0.068810 to 0.068610 on

the T.REF file. (As another test you may keep 0.068810 but change the

hkl for this line from 112 to 121 and run it once again as bescribed

below.)

>PIRUM

>IP

to 2

>RETURN

>PREAD

give T.REF

>SYS

to 3

>COND

(remember that HELP command is available)

>INP

(HELP command is available also on this level)

give

>h0l : h+l=2n

and

>0kl : k=2n

>RETURN

>EXIT

>RUN

Give a list file name TTT.LST

>EXIT

Compare the output lists TTT.LST and T2.LST and note the warning

message on the TTT.LST (where abs(2theta-obs minus 2theta-calc) is

more than the allowed error window 0.03 deg.)

Now you may begin to understand the flexibility of the program.

Some useful properties are CYCLES=0 and a wide error window (i.e. an

increased DT parameter), for preliminary tests...

The reason Hess weigths are used is that ICDD seems to prefer

equal weights on all 2-theta values. (It also makes the program

more in agreement with the NBS*AIDS83 program by Mighell, Hubbard

and Stalik, 1981.) One may have different opinions about this as

it may be dependent on data collection procedures, intensity

variations, overlaps in the high angle region, variations in line

widths etc., but as Hess weigths usually give larger standard

deviations, it may in general be more safe to use them.